HEALTH AND M EDICINE	Copyright © 2017

The Authors, some

# Effective drug combination for Caenorhabditis elegans nematodes discovered by output-driven feedback system control technique

Xianting Ding,1*† Zach Njus,2† Taejoon Kong,2 Wenqiong Su,1 Chih-Ming Ho,3 Santosh Pandey2*

Infections from parasitic nematodes (or roundworms) contribute to a significant disease burden and productivity losses for humans and livestock. The limited number of anthelmintics (or antinematode drugs) available today to treat these infections are rapidly losing their efficacy as multidrug resistance in parasites becomes a global health challenge. We propose an engineering approach to discover an anthelmintic drug combination that is more potent at killing wild-type Caenorhabditis elegans worms than four individual drugs. In the experiment, freely swimming single worms are enclosed in microfluidic drug environments to assess the centroid velocity and track curvature of worm movements. After analyzing the behavioral data in every iteration, the feedback system control (FSC) scheme is used to predict new drug combinations to test. Through a differential evolu- tionary search, the winning drug combination is reached that produces minimal centroid velocity and high track curvature, while requiring each drug in less than their EC50 concentrations. The FSC approach is model-less and does not need any information on the drug pharmacology, signaling pathways, or animal biology. Toward com- bating multidrug resistance, the method presented here is applicable to the discovery of new potent combina- tions of available anthelmintics on C. elegans, parasitic nematodes, and other small model organisms.

rights reserved; exclusive licensee American Association for the Advancement of Science. No claim to original U.S. Government Works. Distributed under a Creative Commons Attribution NonCommercial License 4.0 (CC BY-NC).

Downloaded from http://advances.sciencemag.org/ on August 28, 2018

INTRODUCTION

Resistance of pathogens to pharmaceutical drugs is a widely acknowl- edged problem in medicine (1–7). Parasitic nematodes are a classic ex- ample of multidrug resistance, because these animals have evolved resistance to chemotherapy through successive generations (8–13). Neglected tropical diseases arising from parasitic nematode infections contribute to extensive disease burden and socioeconomic losses, par- ticularly in the underdeveloped countries of the world (14–18). There are a number of contributing factors for the emergence of multidrug resistance in parasitic nematodes. For instance, there are a limited num- ber of drugs approved by the World Health Organization (WHO), the cost and time for new drug discovery are astounding, existing chemo- therapy options require multiple treatments and doctor visits, large- scale deworming efforts on millions of infected people each year is a daunting task, and there is a large geographic variation in the nature and dynamics of parasitic infection (19–26).

The general classes of compounds, called anthelmintics, have been developed to treat infections by parasitic nematodes (13, 21, 27). One class of anthelmintics approved for use by WHO is the nicotinic acetyl- choline receptor (nAChR) agonist, which can be further divided into two targeted receptor subtypes: L-type and N-type (27). Common ex- amples of L-type nAChR agonists are levamisole and pyrantel, whereas an example of an N-type AChR agonist is methyridine (21, 27, 28). Tri- bendimidine, a relatively understudied drug, is the only anthelmintic that is undergoing phase 4 clinical testing in China and shows promise against a broad range of parasitic nematode infections (4, 5). With the repertoire of present-day anthelmintics, it would be difficult to imagine

1State Key Laboratory of Oncogenes and Related Genes, Institute for Personalized Medicine, School of Biomedical Engineering, Shanghai Jiao Tong University, Shanghai, China. 2Department of Electrical and Computer Engineering, Iowa State University, Ames, IA 50011, USA. 3Department of Mechanical and Aerospace Engi- neering, University of California, Los Angeles, Los Angeles, CA 90095, USA.

*Corresponding authors: Email: dingxianting@sjtu.edu.cn (X.D.); pandey@iastate. edu (S.P.)

†These authors contributed equally to this work.

that parasitic nematodes and their associated infections continue to be a pressing issue to the society. However, challenges exist in both the fron- tiers of drug discovery and testing. Besides higher throughput com- pound screening assays in the laboratory, we need methods to monitor and quantify resistance of nematodes in the field, cost-efficient ways to translate basic research to field testing, and models to predict interactions between multiple drugs when used in rotation or in com- bination (18, 29–33).

Because of the apparent challenges in conducting large-scale drug tests on parasitic nematodes, researchers have often resorted to the free-living Caenorhabditis elegans to carry out experiments to determine novel drug targets or molecular modes of action and resistance (33, 34). Among the many benefits, C. elegans is easy to culture on agarose plates, has a short life span, is amenable to genetic and mutagenesis tools, and exhibits genetic tractability through generations (35, 36). Technologies, such as optofluidics and microfluidics, have been developed around

C. elegans to test how drug compounds affect their viability and

movement, detect effects of toxins on their behavior and life cycle, and decipher the role of individual genes in producing a specific behav- ioral response (36–40). High-resolution imaging tools with sophisticated worm tracking software have been realized to automatically generate data on behavioral phenotype, thus minimizing the role of manual screening (41–43). Technological advancements have definitely improved the pace of drug testing with the ability to screen thousands of small-molecule compounds for effectiveness in C. elegans (44, 45). However, the process of drug discovery still relies on the conventional approach of searching for suitable molecular targets and understanding the modes of action of a new compound (34). In an attempt to address this bottleneck, we questioned whether a control engineering approach could be used to speed up the process of drug discovery wherein an algorithmic search helps us achieve the desired drug effectiveness against parasitic nematode infections (46).

Here, we show that a specific combination of four anthelmintics (namely, levamisole, pyrantel, methyridine, and tribendimidine) is more effective at killing wild-type C. elegans than each of the four anthelmintics

Downloaded from http://advances.sciencemag.org/ on August 28, 2018

used separately. A schematic representation of our experiment is illustrated in Fig. 1. Active worms are placed in microfluidic chambers where they are free to swim around in a drug environment. Their move- ment behavior is recorded in response to various concentrations of the four anthelmintics. The dose-response data for individual drugs are fed into a feedback system control (FSC) optimization scheme (46–48) that suggests new combinations of the four drugs to test during each iteration. Using a custom worm tracking program, we record the move- ment coordinates of single worms at real time. We found that two movement parameters, average centroid velocity and track curvature, adequately represent how certain drug combination affects the worm. Then, through multiple iterations of testing drug combinations, the FSC technique is able to produce the winning cocktail that is more potent but uses less than the median effective concentration (EC50) values of the four anthelmintics.

RESULTS

Dose response of individual drugs

To start with, four anthelmintic drugs are chosen that are known to act on the C. elegans worms (49–53). Different concentrations of each drug are prepared in M9 buffer. For each concentration, the microfluidic chamber is filled with the drug solution and worm behavior is recorded for 600 s at one frame per second. This time duration is sufficiently long to observe the drug-induced degradation in the worm’s physical ability (54–57). The average velocity of the worm’s centroid in an experimen- tal run is calculated and normalized to that obtained in control ex- periments with M9 buffer to obtain a percentage response (that is, dose response). Each drug concentration is tested on at least three separate chips with one worm per chamber and three chambers per chip. Figure 2 shows the dose response of C. elegans to the four drugs from which the EC50 value is calculated (that is, drug concentration where the percentage dose response is 50%). The EC50 values obtained

Fig. 1. Schematic representation of the experiment. The effect of the four anthel- mintics (namely, levamisole, pyrantel, methyridine, and tribendimidine) is tested on active, wild-type C. elegans. The dose-response data are fed into an FSC scheme that suggests new combinations of the four anthelmintics to test for worm survivability. Through multiple iterations of trying nonoptimal combinations and using a directed evolutionary search, the winning cocktail is eventually reached that makes the worm inactive, as judged by the values of average centroid velocity and track curvature.

from our experiments are as follows: levamisole, 2.23 mM; pyrantel,

107.5 mM; tribendimidine, 4.68 mM; and methyridine, 1672 mM. From the measured EC50 values, it appears that levamisole is the most potent drug, whereas methyridine is the least potent one.

FSC implementation

After obtaining the dose response of the four drugs, we proceed to use the FSC technique to discover a new combination that is more potent than the EC50 concentrations of any individual drug. In other words, we want to check the possibility of creating a concoction using the four drugs at reduced concentrations but with higher potency or greater ef- ficacy on worm activity (that is, lower average centroid velocity, typically

<15 mm/s). This will allow us to use lower drug dosages, thereby reducing

the associated costs. To use the FSC technique, we choose six concentra- tion keys for each of the four drugs. A concentration key refers to a rep- resentative, integer value assigned for an actual concentration of a drug. The FSC technique does not use actual drug concentrations as input but rather requires concentration keys to determine the drug combinations for the subsequent steps. The concentration keys are chosen as follows: The highest concentration key is the EC50 value of the drug. The next lower concentration key corresponds to half of the present concentra- tion. The concentration keys and their respective drug concentrations are listed with specific color codes for each drug (Fig. 3, A and B).

For the first iteration, all the concentration keys are inputted into the FSC program, which then outputs eight drug combinations to test (that is, hereby called P1 to P4 and T1 to T4). Figure 3C illustrates the process flow where a drug combination is represented by a set of four unique numbers denoting the concentration keys of the drugs used according to Fig. 3 (A and B). As an example, P1 in iteration 1 is represented as “4524”, which indicates the following concoction: levamisole, 1 mM; tri- bendimidine, 4 mM; pyrantel, 12.5 mM; and methyridine, 800 mM. Each drug combination is prepared, filled in the microfluidic chambers, and tested on single L4-stage C. elegans. Each drug combination is tested on at least three to five different chips, with each chip having three separate chambers with one worm per chamber. From the recorded videos, the average centroid velocity is obtained and normalized to values from the

Fig. 2. Dose response of the four anthelmintics is shown. Here, the percentage response is obtained by calculating the average centroid velocity of a worm pop- ulation at a certain drug concentration and normalizing it to the velocity from the control experiments with M9 buffer. The EC50 (that is, drug concentration where the percentage response is 50%) values obtained from our microfluidic device are as follows: levamisole, 2.23 mM; pyrantel, 107.5 mM; tribendimidine, 4.68 mM; and methyridine, 1672 mM. Each drug concentration is tested on at least three separate chips with one worm per chamber and three chambers per chip.

Fig. 3. Drug concentration keys and FSC iterative process flow. (A) The con- centration keys (0 to 5) are assigned to six individual concentrations of each anthelmintic. The highest concentration key (that is, key 5) is chosen close to the EC50 values, thus limiting our search to a winning combination having no more than the EC50 value of each compound. (B) Distinct color codes are chosen

Fig. 4. Microfluidic device and worm tracking. (A) Movement of a wild-type C. elegans worm is tracked through a 600-s video where it shows distinct periods of being active, semiactive, and inactive. An active worm has relatively high velocity and smaller curvature, whereas an inactive worm displays minimal velocity and larger curvature. (B) Tracks of the same worm [as in (A)] are shown during the active, semi- active, and inactive periods (i to iii). The active worm has relatively straight tracks with a smaller curvature, whereas the inactive worm shows considerable struggle with minimal displacement and a larger curvature. Scale bars, 400 mm.

Downloaded from http://advances.sciencemag.org/ on August 28, 2018

for each drug for visual representation. (C) In each iteration, the eight combina- tions are grouped into P group (P1 to P4) and T group (T1 to T4) candidates. The first iteration tests eight combinations. The second iteration retains the four best combinations from the first iteration as the P group and suggested four new com- binations as the T group. This procedure is repeated in each iteration until the winning combination is reached (T3 in the fourth round here).

control experiments, and the percentage dose response (or performance parameter) is calculated for all the eight drug combinations.

For the second iteration, the performance of the P and T groups from the first iteration is compared (that is, P1 versus T1 and P2 versus T2, etc.). The four better-performing drug combinations are retained as P1 to P4, and the four lower-performing drug combinations are dis- carded. In addition, the FSC program suggests four new combinations as T1 to T4 that are also tested along with the P group in the second iteration. The experimental process is repeated where the eight drug combinations are prepared and tested on C. elegans within microfluidic chambers. The performance of the eight combinations is evaluated to determine whether the effective or winning drug combination is reached. Otherwise, the above procedure is repeated for successive itera- tions until the winning drug combination is achieved. Figure 3C shows the P and T group drug combinations for every iteration. In our case, the winning combination was reached after the fourth iteration.

Average centroid velocity and track curvature

From our initial observations, it was evident that worms in the drug environment showed three distinct categories of movement activity (that is, active, semiactive, and inactive). In a potent drug solution, worms initially show unrestricted swimming patterns (active period) that slowly change to restricted swimming with more struggle (semi- active period) and eventually to an almost immobile posture (inactive period). Figure 4A shows the superimposed tracks of a single worm in a potent drug solution through 600 s of recording with its three periods of activity which are color coded for visual ease. In this example, the active worm exhibits relatively high velocity (134.44 mm/s) and smaller

track curvature (9.13 mm−1), the semiactive worm shows medium ve- locity (99.21 mm/s) and track curvature (15.84 mm−1), whereas the in- active worm displays minimal velocity (5.5 mm/s) and larger track curvature (56.66 mm−1). Figure 4B shows the tracks of the same worm plotted separately during the active, semiactive, and inactive periods. An active worm moves in a nearly straight track leading to a smaller value of track curvature, whereas an inactive worm struggles considerably with minimal body displacement yielding a larger value of track curvature. The average centroid velocity and track curvature obtained from the four rounds of iterations are plotted in Fig. 5 (A and B), respectively. Each control and experimental drug combination is tested on at least three to five different chips, with each chip having three separate chambers with one worm per chamber. The data plotted are only from the final 120 s of each video recording that lasted 600 s, although the data averaged over the entire 600 s showed similar trends (data not shown). It is interesting to compare the velocity and curvature data alongside each other. The first iteration that proposed eight drug com- binations led to four desirable combinations (that is, P1, P2, T3, and T4) for the second iteration. Unfortunately, the T group performed worse than the P group in the second iteration. The third iteration found four desirable combinations (that is, P1, P3, P4, and T2) that were carried over to the fourth iteration. The T group performed much better than the P group in the fourth iteration, particularly T3 that is chosen as the

effective or winning drug combination.

We found that both the centroid velocity and track curvature data are needed to adequately represent the movement behavior of the worms. These data can either be instantaneous or averaged over a cer- tain time period. Figure 6 shows another way to view the data sets. Here, we plot the instantaneous velocities and curvature values for eight representative drug combinations to show the general trends we observed. Some combinations have very high velocity (>100 mm/s) and low curvature (<25 mm−1), indicating that the drugs have a negligible effect on worm movement within the observed time period (Fig. 6, D to F). Other combinations may have medium velocity (50 to 100 mm/s)

Downloaded from http://advances.sciencemag.org/ on August 28, 2018

Fig. 5. Average centroid velocity and track curvature. (A) The centroid velocity averaged over all the worms for a given drug combination is plotted through all the four iterations. The centroid velocity of a single worm is averaged over the last 120s of the 600-s duration of the worm exposure. (B) Similarly, the track curvature averaged over all the worms for a given drug combination is depicted through the four iterations. Centroid velocity and curvature of worms in control experiments using M9 buffer are also shown. The winning combination was chosen as T3 in the fourth iteration that produced minimal centroid velocity and a larger curvature. Each drug combination is tested on at least three to five different chips. Each chip has three separate chambers with one worm per chamber. Bars are SEM.

and curvature (25 to 50 mm−1), suggesting that the drugs are affecting the worms but better alternatives are still possible. The winning combi- nation has minimal velocity (<10 mm/s) and large curvature (>50 mm−1) throughout the 600 s (Fig. 6A). We saw that the winning drug combi- nation started paralyzing the worm within the first minute itself, and the decreased movement was sustained through the 600 s of recording. This suggested that the length of each experiment could be curtailed to with- in 2 min, and the movement parameters (that is, instantaneous velocity and curvature data) could be used as a rapid indicator of potency.

The relationship between centroid velocity and track curvature as measured in the FSC technique can be illustrated as a two-dimensional plot (Fig. 7). The color codes in the plot represent the worm in the three periods of activity: active, semiactive, and inactive. In this depiction, we are interested in potent combinations that make the worm inactive with minimal velocity and larger curvature. In iteration 1, two combi- nations (P1 and T3) induce inactivity in the worm, P3 makes the worm semiactive, and the remaining combinations do not seem to affect the movement of the active worm (Fig. 7A). Similarly, iteration 2 yields P1 as a good candidate, P3 makes the worm semiactive, whereas the rest of the combinations do not show an effect on the movement of the active worm (Fig. 7B). Iteration 3 produces no combination that makes the worm inactive; however, four combinations (P1, P3, P4, and T2) make the worm semiactive (Fig. 7C). Iteration 4 yields two combina- tions (T1 and T3) that induce inactivity in the worm, we chose T3 as the winning drug combination that has the minimum centroid velocity (Fig. 7D). The winning combination (T3 of fourth iteration) consists of 1 mM levamisole, 4 mM tribendimidine, 12.5 mM pyrantel, and 100 mM methyridine.

Fig. 6. Instantaneous centroid velocity and track curvature are plotted for the entire 600 s of drug exposure. The data at every second are obtained by averaging the velocity or curvature values for all the worms exposed to a certain drug combi- nation. (A) The winning combination, T3 of the fourth iteration, has minimal velocity (<10 mm/s) and large curvature (>50 mm−1) throughout the 600 s. (B and C) These combinations show medium velocity (50 to 100 mm/s) and curvature (25 to 50 mm−1), suggesting that the compounds affect the worms, but better alternatives are still possible. (D to F) These combinations exhibit high velocity (>100 mm/s) and low cur- vature (<25 mm−1), indicating that the drugs have a negligible effect on worm move- ment within the observed time period.

DISCUSSION

We showed that an optimal combination of four anthelmintics can be found that uses much less than the EC50 concentrations of each drug but is more potent than any of them. The experimental data, gathered from the movement dynamics of a single C. elegans enclosed in micro- fluidic chambers, were fed into an FSC optimization scheme. Through a differential evolution search and feedback calculation process, the algorithm produced sets of drug combinations to test in each iteration that eventually led to the winning combination. The interplay of the three technologies (that is, microfluidics, worm image tracking, and FSC search) was crucial to the successful demonstration of the winning drug combination. Microfluidic devices enabled us to record the behavior of freely swimming worms in enclosed chambers. The worm tracking program allowed us to process the recorded videos and extract the average centroid velocity and track curvature for every experiment. The FSC technique provided the means to predict new combinations and optimize our search with every iteration. In our knowledge, this is the first use of microfluidics to discover effective drug combinations on a small-animal model organism through a sys- tematic FSC search. Furthermore, this work validates the workability of the FSC search technique on C. elegans model organisms beyond its current usage to cell-based systems (for example, in cancer cell viabil- ity, viral infection, stem cell maintenance and differentiation, and en- dothelial cell viability) (46–48).

Our rationale for the choice of the four drugs was based on our pre- vious experiments (37). From analyzing the behavioral patterns of worm paralysis in individual drugs, we found that tribendimidine

Downloaded from http://advances.sciencemag.org/ on August 28, 2018

Fig. 7. Two-dimensional plot of track curvature versus centroid velocity. In the drug environment, the worm is classified as active, semiactive, or inactive as demon- strated in Fig. 4, and the plot is accordingly segregated into the three areas of activity. We are primarily interested in combinations that induce inactivity in the worm. (A) In iteration 1, two combinations (that is, P1 and T3) are able to make the worm inactive. Besides in P3, the active worm does not succumb to any other combination and is classified as being active. (B) In iteration 2, only P1 makes the worm inactive. The rest of the combinations do not seem to affect the active worm. (C) In iteration 3, the worm is in a semiactive or active state for all combinations. (D) In iteration 4, three combina- tions (T1, T2, and T3) are able to make the worm inactive, and the winning combination is chosen as T3.

was more potent than levamisole or pyrantel in causing irreversible pa- ralysis, methyridine acted on a different subtype of nAChR receptors than the other three anthelmintics, and pyrantel was more effective than levamisole, although they both had similar modes of action (17). Thus, on the basis of differences in phenotype and paralysis observed earlier, we started with these four anthelmintics for our FSC-guided search for optimal drug combinations. With every possibility, there may be other drug compounds that lead to concoctions with better potency than that reported here.

The FSC search technique relies on the user’s judgment to choose

the most suitable drug library (for example, preferably with distinct mechanisms, molecular targets, or measured effects) for the desired outcome. However, a major challenge in choosing an appropriate drug library lies in the fact that the number of drug combinations increases exponentially with increasing number of drugs or doses. As an example, assume that the drug library consists of “n” drugs with “m” concentra- tions of each drug to be tested. Each concentration is further tested at least three times for consistency. This would involve conducting (m^n) × 3 experiments. Here, we used n = 4 drugs, m = 6 concentration keys, and 9 to 12 worms for each concentration, which gave a total of at least 10,000 single worm experiments to run. If we chose more number of drugs, the number of experiments would be overwhelming with conventional methods. Therefore, the choice of the initial drug library could involve drug candidates that cover multiple signaling pathways and potentially contribute to the end point system readouts. However, if the initial drug library is too large, FSC takes longer iterations to downselect those nonsignificant drugs.

The FSC search technique isa powerful tool toward drug discovery. It is a model-less approach that does not require information about the drug’s pharmokinetics or the genetics/metabolomics of the animal un- der study (as opposed to models based on big data and machine learning). No information is needed about how drug signaling pathways work or how multiple drugs interact in combinations. This model-less approach is advantageous in scenarios where data sets are limited in size and experiments are time-consuming. Here, the assumption is that we do not need a complete model of the complex biosystem (that is, C. elegans) to produce a desired output from input variables. The FSC technique uses the system’s output (that is, movement data) as the feedback criteria to optimize the input values (that is, individual drug concentrations), eventually reaching the desired output for the winning drug combination.

We demonstrated a specific combination of four anthelmintics that uses drug dosages lower than the individual EC50 values of each drug but is still much more potent on C. elegans than any of these drugs administered individually. The experiment involves recording the movement of freely swimming single C. elegans in microfluidic cham- bers for 600 s, along with a tracking program to automatically extract the average centroid velocity and track curvature. The EC50 values measured are 2.23 mM for levamisole, 107.5 mM for pyrantel, 4.68 mM for tribendimidine, and 1672 mM for methyridine. Differential evolution is then used to refine the concentrations of individual drugs in the input data sets based on feedback from the worm movement data. Effective drug combination is reached in the fourth iteration with minimal cen- troid velocity (<10 mm/s) and large track curvature (>50 mm−1). The winning combination is composed of 1 mM levamisole, 4 mM tribendimidine,

12.5 mM pyrantel, and 100 mM methyridine. Much reduced movement

ability is achieved by the winning combination consisting of the four anthelmintics at dosage levels lower than their individual EC50 values. The winning combination exhibits high potency as achieved by the maximal dosage of the four drugs but with reduced toxicity and lower cost of drug consumption. The FSC-guided drug combinatorial therapy on a small-animal model discussed here shows that a purely engineering approach for multidrug resistance in nematodes is possible with relevance to other model organisms and parasites.

MATERIALS AND METHODS

Microfluidic        chip        design The microfluidic device has been previously used to measure the tox- icity of C. elegans to liquid cyanide solutions (18). Briefly, the chip was designed in AutoCAD to have three identical microfluidic chambers (Fig. 2). The chambers were capsule-shaped and without any sharp edges to allow for the smooth movement of C. elegans. The input port was tapered leading into the device to permit easy injection of worms into the chamber but prevent the exit of worms. Three chambers can be accommodated within the microscope’s field of view and imaged sim- ultaneously. The chip design was sent to an outside vendor (FineLine Imaging) for printing the physical masks. After the masks were ob- tained, an ultraviolet-sensitive polymer, SU-8, was spin-coated on a 3-inch silicon wafer to a thickness of about 80 mm. The patterns on the physical mask were transferred onto the SU-8, which is then de- veloped. A polydimethylsiloxane (PDMS) polymer was put on the developed SU-8 layer, allowed to dry at room temperature, and later peeled off the SU-8 layer. Subsequently, access holes were punched in the PDMS layer, which is irreversibly bonded to a glass slide to create the microfluidic chip.

Downloaded from http://advances.sciencemag.org/ on August 28, 2018

Imaging setup and real-time worm tracking

Postfabrication, the chip was secured on the stage of a Leica MZ16 transmission stereozoom microscope using a plastic tape. Individual chambers were filled with M9 buffer or drug solution using a Tygon microbore tubing (inner diameter, 0.51 mm; outer diameter, 1.52 mm) connected to a 5-ml syringe. Single L4-stage C. elegans were picked using a sterilized worm picker and dropped on the input ports of the microfluidic chambers. By applying a small amount of pressure through the syringe, individual worms entered the chamber and start swimming in the liquid medium. All experiments were performed at 20° to 22°C using L4-stage worms. Earlier, wild-type C. elegans were grown on nutrient growth medium agarose plates with Escherichia coli OP50 bacterial food. The microscope was connected to a high-speed QICAM 12-bit Mono Fast 1394 cooled digital camera with QCapture Pro software, which allows us to record worm behavior across three micro- fluidic chambers at real time. Digital images (1392 × 1040 pixels) were captured from the recording system for 600 s at one frame per second and were later appended and stitched into single or multiple AVI files for further analyses.

A custom worm tracking program was written in C++ pro- gramming language. The program reads the input video file, segments the video into a series of images, identifies the worm body from the background, and then records the x-y coordinates of the body’s centroid through all the images of the video. The program was able to track worms moving forward or backward, flailing, paused intermittently or indefinitely, and touching other worms or edges of microfluidic de- vices. In addition, the program adequately keeps track of the worm’s boundary throughout the video as it freely moves within the microflui- dic chamber, which is often difficult if the worm collides with or touches with other objects in the vicinity. The tracked centroid velocities of in- dividual worms were outputted into an Excel workbook in the form of lists of x-y coordinates as a function of time. Statistical analysis and dose-response curve fitting of the generated data were performed using GraphPad Prism. In this software, the steps involve entering the data points, transforming the x axis values to log scale, and normalizing the y axis values. Then, the “dose response” option was selected followed by the “nonlinear regression” for curve fitting. The prism provides op- tions for the available types of nonlinear regression fits, and the “least square fit” is chosen. The centroid data were used to calculate the instan- taneous velocity and average velocity of the worms. In addition, we es- timated the average curvature of the worm tracks from the different experiments using the Menger curvature formulation (58).

Using microfluidics and automated worm tracking software, the average centroid velocity and track curvature were suitable param- eters to score drug-induced inactivity in C. elegans. The signatures of the track path (such as velocity, distance moved, and track cur- vature) correlate with the degradation of the worm’s physical abil- ity (54–57). Because the track paths were recorded at a relatively lower resolution and frame rate, it was possible to observe the be- havior of multiple worms within the field of view, which increases the throughput. In comparison, other health metrics related to the body posture (such as thrashing rate, bending frequency, amplitude, and wavelength) were recorded on single worms at a much higher resolution and frame rate, and have low correlation to drug-induced inactivity of C. elegans (54–57). Manual scoring of drug-induced death of C. elegans by applying a stimulus to induce motion was feasible on a macroscopic plate or inhibition assays. However, there were definite benefits of using the tracking software over manual scoring of worm death because it provides a means to track the reduced velocity at real

time with the advantages of data reproducibility, reduced labor, and elimination of human bias during data collection (56, 57).

FSC         search         technique The FSC search technique adopted in this study was according to earlier publications (46–48). Figure 1 demonstrates this FSC optimization pro- cess flow. First, the dose response of C. elegans to the four chosen drugs was conducted. Then, the FSC technique was applied that consists of the following steps: (i) Drug combination pool with four random combina- tions (P group) were generated and experimentally tested. (ii) Feedback search algorithm named differential evolution (DE) was adopted to gen- erate another four potentially better drug combinations to be tested in the next round of experiments (T group). DE simulates the natural evo- lution process by applying “mutation” to each drug combination in the P group (low percentage alterations at a few drug doses) and “crossover” between drug combinations within the P group (replacing a portion of one drug combination with another drug combination). The efficacy of each combination in the T group was also experimentally evaluated. The winning drug combinations from the P and T groups were retained to update the P group. The updated P group goes through DE calcu- lation again to generate another T group for comparison. As a result, the P group evolves to be more and more potent (46, 47). (iii) The DE- suggested drug combinations for the four original drug combinations were tested for their antiworm efficacy. The superior ones were used to update the drug combination pool (P group). (iv) Feedback calcula- tion and experimental evaluation described in the steps (i to iii) were repeated until the desired drug combination was identified. The FSC technique was programmed with the MATLAB software. In each iteration, we fed the program with the drug combination’s composition and the efficacy of both the P and T groups. The program updates the P group and generates a new T group to be tested in the next iteration.

REFERENCES AND NOTES

- M. Boelaert; NIDIAG Consortium, Clinical research on neglected tropical diseases: Challenges and solutions. PLOS Negl. Trop. Dis. 10, e0004853 (2016).

- G. Zilahi, A. Artigas, I. Martin-Loeches, What’s new in multidrug-resistant pathogens in the

ICU? Ann. Intensive Care 6, 1–11 (2016).

- C. L. Ventola, The antibiotic resistance crisis part 1: Causes and threats. Pharm. Ther. 40, 277–283 (2015).

- R. Penchovsky, M. Traykovska, Designing drugs that overcome antibacterial resistance: Where do we stand and what should we do? Expert Opin. Drug Discovery 10, 631–650 (2015).

- G. Gualano, S. Capone, A. Matteelli, F. Palmieri, New antituberculosis drugs: From clinical trial to programmatic use. Infect. Dis. Rep. 8, 6569 (2016).

- R. Kaminsky, P. Ducray, M. Jung, R. Clover, L. Rufener, J. Bouvier, S. S. Weber, A. Wenger,

S. Wieland-Berghausen, T. Coebel, N. Gauvry, F. Pautrat, T. Skripsky, O. Froelich,

C. Komoin-Oka, B. Westlund, A. Sluder, P. Mäser, A new class of anthelmintics effective against drug-resistant nematodes. Nature 452, 176–180 (2008).

- D. O’Connell, Worming a way to better drugs. Nat. Rev. Microbiol. 4, 568–569 (2006).

- R. K. Prichard, M.-G. Basáñez, B. A. Boatin, J. S. McCarthy, H. H. García, G.-J. Yang, B. Sripa,

S. Lustigman, A research agenda for helminth diseases of humans: Intervention for control and elimination. PLOS Negl. Trop. Dis. 6, e1549 (2012).

- N. de Silva, B.-N. Ahmed, M. Casapia, H. J. de Silva, J. Gyapong, M. Malecela,

A. Pathmeswaran, Cochrane reviews on deworming and the right to a healthy, worm-free life. PLOS Negl. Trop. Dis. 9, e0004203 (2015).

- T.-W. Jia, S. Melville, J. Utzinger, C. H. King, X.-N. Zhou, Soil-transmitted helminth reinfection after drug treatment: A systematic review and meta-analysis. PLOS Negl. Trop. Dis. 6, e1621 (2012).

- L. F. V. Furtado, A. C. P. de Paiva Bello, É. M. L. Rabelo, Benzimidazole resistance in helminths: From problem to diagnosis. Acta Trop. 162, 95–102 (2016).

- R. G. Woodgate, R. B. Besier, Sustainable use of anthelmintics in an Integrated Parasite Management Program for sheep nematodes. Anim. Prod. Sci. 50, 440–443 (2010).

Downloaded from http://advances.sciencemag.org/ on August 28, 2018

L. C. Gasbarre, L. L. Smith, E. Hoberg, P. A. Pilitt, Further characterization of a cattle nematode population with demonstrated resistance to current anthelmintics. Vet. Parasitol. 166, 275–280 (2009).

- M. Bangert, D. H. Molyneux, S. W. Lindsay, C. Fitzpatrick, D. Engels, The cross-cutting contribution of the end of neglected tropical diseases to the sustainable development goals. Infect. Dis. Poverty 6, 73 (2017).

- C. Liu, R. Luo, H. Yi, L. Zhang, S. Li, Y. Bai, A. Medina, S. Rozelle, S. Smith, G. Wang, J. Wang, Soil-transmitted helminths in Southwestern China: A cross-sectional study of links to cognitive ability, nutrition, and school performance among children. PLOS Negl. Trop. Dis. 9, e0003877 (2015).

- T. Allen, M. Parker, Deworming delusions? Mass drug administration in East African Schools. J. Biosocial. Sci. 48, S116–S147 (2016).

- Y. Hu, E. G. Platzer, A. Bellier, R. V. Aroian, Discovery of a highly synergistic anthelmintic combination that shows mutual hypersusceptibility. Proc. Natl. Acad. Sci. U.S.A. 107, 5955–5960 (2010).

- Y. Hu, S.-H. Xiao, R. V. Aroian, The new anthelmintic tribendimidine is an L-type (levamisole and pyrantel) nicotinic acetylcholine receptor agonist. PLOS Negl. Trop. Dis. 3, e499 (2009).

- C. H. King, Health metrics for helminth infections. Acta Trop. 141, 150–160 (2015).

- M. Rosenberg, J. Utzinger, D. G. Addiss, Preventive chemotherapy versus innovative and intensified disease management in neglected tropical diseases: A distinction whose shelf life has expired. PLOS Negl. Trop. Dis. 10, e0004521 (2016).

- H. Qian, A. P. Robertson, J. A. Powell-Coffman, R. J. Martin, Levamisole resistance resolved at the single-channel level in Caenorhabditis elegans. FASEB J. 22, 3247–3254 (2008).

- T. Wang, X.-s. Yuan, M.-B. Wu, J.-P. Lin, L.-R. Yang, The advancement of multidimensional QSAR for novel drug discovery - where are we headed? Expert Opin. Drug Discovery 12, 769–784 (2017).

- M. V. Johansena, T. Lier, P. Sithithaworn, Towards improved diagnosis of neglected zoonotic trematodes using a One Health approach. Acta Trop. 141, 161–169 (2015).

- M. Walker, T. S. Mabud, P. L. Olliaro, J. T. Coulibaly, C. H. King, G. Raso, A. U. Scherrer,

J. R. Stothard, J. C. Sousa-Figueiredo, K. Stete, J. Utzinger, M.-G. Basáñez, New approaches to measuring anthelminthic drug efficacy: Parasitological responses of childhood schistosome infections to treatment with praziquantel. Parasit. Vectors 9, 41 (2016).

- S. J. Campbell, S. V. Nery, J. S. McCarthy, D. J. Gray, R. J. Soares Magalhães,

A. C. A. Clements, A critical appraisal of control strategies for soil-transmitted helminths.

Trends Parasitol. 32, 97–107 (2016).

- E. A. Johnston, J. Teague, J. P. Graham, Challenges and opportunities associated with neglected tropical disease and water, sanitation and hygiene intersectoral integration programs. BMC Public Health 15, 547 (2015).

- H. Qian, R. J. Martin, A. P. Robertson, Pharmacology of N-, L-, and B-subtypes of nematode nAChR resolved at the single-channel level in Ascaris suum. FASEB J. 20, 2606–2608 (2006).

- R. J. Martin, G. Bai, C. L. Clark, A. P. Robertson, Methyridine (2-[2-methoxyethyl]-pyridine]) and levamisole activate different ACh receptor subtypes in nematode parasites: A new lead for levamisole-resistance. Br. J. Pharmacol. 140, 1068–1076 (2003).

- V. Leignel, A. Silvestre, J. F. Humbert, J. Cabaret, Alternation of anthelmintic treatments: A molecular evaluation for benzimidazole resistance in nematodes. Vet. Parasitol. 172, 80–88 (2010).

- F. Beugnet, D. Kerboeuf, J. C. Nicolle, D. Soubieux, Use of free living stages to study the effects of thiabendazole, levamisole, pyrantel and ivermectin on the fine structure of Haemonchus contortus and Heligmosomoides polygyrus. Vet. Parasitol. 63, 83–94 (1996).

- T. Kaletta, M. O. Hengartner, Finding function in novel targets: C. elegans as a model organism. Nat. Rev. Drug Discov. 5, 387–399 (2006).

- W. D. Atchison, T. G. Geary, B. Manning, E. A. VandeWaa, D. P. Thompson, Comparative neuromuscular blocking actions of levamisole and pyrantel-type anthelmintics on rat and gastrointestinal nematode somatic muscle. Toxicol. Appl. Pharmacol. 112, 133–143 (1992).

- S. Gaba, J. Cabaret, C. Sauvé, J. Cortet, A. Silvestre, Experimental and modeling approaches to evaluate different aspects of the efficacy of Targeted Selective Treatment of anthelmintics against sheep parasite nematodes. Vet. Parasitol. 171, 254–262 (2010).

- J. Giacomotto, L. Ségalat, High-throughput screening and small animal models, where are

we? Br. J. Pharmacol. 160, 204–216 (2010).

- T. Stiernagle, Maintenance of C. Elegans, Wormbook: The Online Review of C. Elegans Biology (Wormbook, 2006), pp. 1–11.

- D. R. Albrecht, C. I. Bargmann, High-content behavioral analysis of Caenorhabditis elegans

in precise spatiotemporal chemical environments. Nat. Methods 8, 599–605 (2011).

- R. Lycke, A. Parashar, S. Pandey, Microfluidics-enabled method to identify modes of

Caenorhabditis elegans paralysis in four anthelmintics. Biomicrofluidics 7, 64103 (2013).

- J. N. Saldanha, A. Parashar, S. Pandey, J. A. Powell-Coffman, Multiparameter behavioral analyses provide insights to mechanisms of cyanide resistance in Caenorhabditis elegans. Toxicol. Sci. 135, 156–168 (2013).

- N. Chronis, M. Zimmer, C. I. Bargmann, Microfluidics for in vivo imaging of neuronal and behavioral activity in Caenorhabditis elegans. Nat. Methods 4, 727–731 (2007).

- E. Yemini, T. Jucikas, L. J. Grundy, A. E. X. Brown, W. R. Schafer, A database of

Caenorhabditis elegans behavioral phenotypes. Nat. Methods 10, 877–879 (2013).

- N. A. Swierczek, A. C. Giles, C. H. Rankin, R. A. Kerr, High-throughput behavioral analysis in

C. elegans. Nat. Methods 8, 592–598 (2011).

- B. T. Moore, J. M. Jordan, L. R. Baugh, WormSizer: High-throughput analysis of nematode size and shape. PLOS ONE 8, e57142 (2013).

- M. R. Lamprecht, D. M. Sabatini, A. E. Carpenter, CellProfiler: Free, versatile software for automated biological image analysis. Biotechniques 42, 71–75 (2007).

- A. R. Schwendeman, S. Shaham, A high-throughput small molecule screen for C. elegans

linker cell death inhibitors. PLOS ONE 11, e0164595 (2016).

- T. I. Moy, A. L. Conery, J. Larkins-Ford, G. Wu, R. Mazitschek, G. Casadei, K. Lewis,

A. E. Carpenter, F. M. Ausubel, High-throughput screen for novel antimicrobials using a whole animal infection model. ACS Chem. Biol. 4, 527–533 (2009).

- H. Tsutsui, B. Valamehr, A. Hindoyan, R. Qiao, X. Ding, S. Guo, O. N. Witte, X. Liu, C.-M. Ho,

H. Wu, An optimized small molecule inhibitor cocktail supports long-term maintenance of human embryonic stem cells. Nat. Commun. 2, 167 (2011).

- X. Ding, D. J. Sanchez, A. Shahangian, I. Al-Shyoukh, G. Cheng, C.-M. Ho, Cascade search for HSV-1 combinatorial drugs with high antiviral efficacy and low toxicity. Int. J. Nanomed. 7, 2281–2292 (2012).

- X. Ding, H. Xu, C. Hopper, J. Yang, C.-M. Ho, Use of fractional factorial designs in antiviral drug studies. Qual. Reliab. Eng. Int. 29, 299–304 (2013).

- L. A. Brown, A. K. Jones, S. D. Buckingham, C. J. Mee, D. B. Sattelle, Contributions from Caenorhabditis elegans functional genetics to antiparasitic drug target identification and validation: Nicotinic acetylcholine receptors, a case study. Int. J. Parasitol. 36, 617–624 (2006).

- E. Ruiz-Lancheros, C. Viau, T. N. Walter, A. Francis, T. G. Geary, Activity of novel nicotinic anthelmintics in cut preparations of Caenorhabditis elegans. Int. J. Parasitol. 41, 455–461 (2011).

- H. Bjørn, A. Roepstorff, P. J. Waller, P. Nansen, Resistance to levamisole and cross-resistance between pyrantel and levamisole in Oesophagostomum quadrispinulatum and Oesophagostomum dentatum of pigs. Vet. Parasitol. 37, 21–30 (1990).

- R. J. Martin, A. P. Robertson, Mode of action of levamisole and pyrantel, anthelmintic resistance, E153 and Q57. Parasitology 134, 1093–1104 (2007).

- S. R. Lockery, S. E. Hulme, W. M. Roberts, K. J. Robinson, A. Laromaine, T. H. Lindsay,

G. M. Whitesides, J. C. Weeks, A microfluidic device for whole-animal drug screening using electrophysiological measures in the nematode C. elegans. Lab Chip 12, 2211–2220 (2012).

- J.-H. Hahm, S. Kim, R. DiLoreto, C. Shi, S.-J. V. Lee, C. T. Murphy, H. G. Nam, C. elegans maximum velocity correlates with healthspan and is maintained in worms with an insulin receptor mutation. Nat. Commun. 6, 8919 (2015).

- A. Bansal, L. J. Zhu, K. Yen, H. A. Tissenbaum, Uncoupling lifespan and healthspan in

Caenorhabditis elegans longevity mutants. Proc. Natl. Acad. Sci. U.S.A. 112, E277–E286 (2015).

- C. J. Cronin, J. E. Mendel, S. Mukhtar, Y.-M. Kim, R. C. Stirbl, J. Bruck, P. W. Sternberg, An automated system for measuring parameters of nematode sinusoidal movement. BMC Genet. 6, 5 (2005).

- D. Ramot, B. E. Johnson, T. L. Berry Jr., L. Carnell, M. B. Goodman, The Parallel Worm Tracker: A platform for measuring average speed and drug-induced paralysis in nematodes. PLOS ONE 3, e2208 (2008).

- J. C. Leger, Menger curvature and rectifiability. Ann. Math. 149, 831–869 (1999).

Acknowledgments: We thank J. Powell-Coffman for her insights in the C. elegans experiments. Funding: This work was partially supported by the State Key Laboratory of Robotics open grant (2016-006 to X.D.) and the China Science and Technology Innovation Zone (17-163-15-XJ-002-002-09 to X.D.). This work was also partially supported by the U.S. National Science Foundation (IDBR-1556370 and CBET-1150867 to S.P.) and the Defense Threat Reduction Agency (HDTRA1-15-1-0053 to S.P.). Author contributions: X.D., C.-M.H., and S.P. conceived the original concept. X.D., C.-M.H., Z.N., and S.P. supervised the project. X.D., W.S., and C.-M.H. conducted the algorithmic searches. Z.N. and T.K wrote the tracking software and collected the behavioral data. X.D., Z.N., T.K., W.S., C.-M.H., and S.P. analyzed the data. X.D. and S.P. wrote the manuscript with inputs from all the other authors. All authors discussed the results and contributed to the revisions. Competing interests: C.-M.H. is the inventor on a patent related to this work (U.S. patent no. 8,232,095B2, dated 31 July 2012). The authors declare no other competing interests. Data and materials availability: All data needed to evaluate the conclusions in the paper are present in the paper. Additional data related to this paper may be requested from the authors.

Submitted 16 June 2017

Accepted 13 September 2017

Published 4 October 2017 10.1126/sciadv.aao1254

Citation: X. Ding, Z. Njus, T. Kong, W. Su, C.-M. Ho, S. Pandey, Effective drug combination for Caenorhabditis elegans nematodes discovered by output-driven feedback system control technique. Sci. Adv. 3, eaao1254 (2017).

Effective drug combination for Caenorhabditis elegans nematodes discovered by output-driven feedback system control technique

Xianting Ding, Zach Njus, Taejoon Kong, Wenqiong Su, Chih-Ming Ho and Santosh Pandey

Sci Adv 3 (10), eaao1254.

DOI: 10.1126/sciadv.aao1254

ARTICLE TOOLS	http://advances.sciencemag.org/content/3/10/eaao1254

REFERENCES

This article cites 57 articles, 2 of which you can access for free http://advances.sciencemag.org/content/3/10/eaao1254#BIBL

Downloaded from http://advances.sciencemag.org/ on August 28, 2018

PERMISSIONS	http://www.sciencemag.org/help/reprints-and-permissions

Use of this article is subject to the Terms of Service

Science Advances (ISSN 2375-2548) is published by the American Association for the Advancement of Science, 1200 New York Avenue NW, Washington, DC 20005. 2017 © The Authors, some rights reserved; exclusive licensee American Association for the Advancement of Science. No claim to original U.S. Government Works. The title Science Advances is a registered trademark of AAAS.