DATA SUMMARY/METADATA - BALTIMORE’S BACK- FRONT YARD GIS DATASET

VERSION 1.0 - 25th June 2024

Alessandro Ossola, PhD – aossola@ucdavis.edu; Dexter Locke, PhD - dexter.locke@usda.gov

DESCRIPTION

This dataset includes the geometry and identity of most of Baltimore’s (MD) residential backyards and front yards (Fig. 1). The Area Of Interest (AOI) includes the entire municipality of Baltimore and refers to the years 2015-2017. Parcel data used to locate yards represent the year 2015, building footprint data represent year 2017 conditions.

Fig. 1 – Example of yard data from Baltimore. Backyard and front yards are coloured in green and yellow, respectively. Violet parcels/yards are corner or other yards not classified in back- or front yards based on a 20 m buffer exclusion zone from each main road intersection.

The algorithm used to locate yards is the same of that proposed in Ossola et al., 2019a, 2019b, with the exception that a 20 m buffer (rather than 15 m) used to exclude corner and other yards/parcels. The new 20 m buffer was selected to account for the greater variation in urban form and morphology found across Baltimore as compared to other cities. This buffer was trialled to apply a more conservative and precise approach to locate back- and front yards. The result is that fewer actual corner yards are misclassified as either front or back yards.

Yard location and identification was performed in ArcGIS Desktop 10.6 as follow (steps from Ossola et al., 2019a):

- The centroid of the largest building in each residential parcel was identified (n=183,315). We assumed the largest structure was the main residential building, as opposed to other non-residential structures (e.g., garage, garden shed).

- For each of those centroids, a line perpendicular to the street centreline (n=34,714) was drawn.

- Then a second perpendicular line to the newly created line was drawn, passing through the building centroid, and extending to the parcel’s lateral boundaries. The second line was used to split each parcel polygon into front and backyards.

- All building footprints within a parcel were further subtracted to define the geometry of yards. The yard strips between the lateral sides of the main residential building and the parcel lateral sides were split evenly between paired front and backyard.

- Residential parcels located within 20m from road intersections were classified as “other parcels/yards”. As these parcels lay at the intersection between two or more streets, a clear distinction between front and backyards in these parcels is not possible. Additionally, some parcels located at a distance of 2-3 properties from some road intersections were misclassified. This occurred mostly in the downtown area in elongated parcels in dense neighbourhoods. Thus, the selection of a 20m buffer excluded yards erroneously located. However, most likely, some yards correctly classified by the algorithm in backyards/front yards were also removed (see RAW data in file YardsALL_buffer20m_RAW.shp).

A total of raw 335,318 yard records were initially located by the GIS algorithm. After removing corner and other yard records by using the 20 m buffer, 258,242 backyards and front yard records were kept in the end (141,511 backyards and 116,731 front yards, see file YardsBF_buffer20m.shp). The number of backyards is higher than front yards as some properties in the downtown area have residential buildings directly facing the street with no or little front yard.

A total of 46,392 “Yard clusters” were also calculated by merging contiguous back- and front yards, respectively (file Yards_clusters_20m.shp) (Fig. 2).

Fig. 2 – Example of clusters of continuous yards from Baltimore derived from data in Fig.1. Backyard and front yard clusters are coloured in green and yellow, respectively.

DATA QUALITY AND ACCURACY

Accuracy of the yard classification algorithm was calculated by visually interpreting randomly-selected yards (n=4,000) and calculating the percentage of yards correctly geolocated and classified as front and backyards. The overall accuracy in locating and identifying Baltimore’s backyards and front yards was 99.05%.

FILES DESCRIPTION

All files are provided in shapefile format.

- house_centroids.shp contains centroids of largest building in each parcel, deemed to be the main house or residence.

- Main_streets.shp contains the street centerlines of the principal roads and excludes alleyways and secondary streets.

- YardsBF_buffer20m.shp contains the geometry of all cleaned and validated 258,242 backyards and front yards across Baltimore. This is the main file to be used in most analyses.

- Yards_clusters_20m.shp contains the location of the clusters of contiguous yards based on the YardsBF_buffer20m.shp file.

- YardsALL_buffer20m_RAW.shp is the raw data from the GIS algorithm. This contains corner and other parcels/yards not included in the final YardsBF_buffer20m.shp file. Some of these excluded yards might have been wrongly excluded, but a more conservative approach was applied to increase overall accuracy of location/classification. Some of these excluded yards could be manually reattributed to the correct yard type.

DATA USE GUIDELINES

Data has been produced by Dr Alessandro Ossola (University of California Davis) and is released under a Creative Commons Attribution-NonCommercial-NoDerivatives 4.0 International License. The data producer give no warranty regarding the data's currency or suitability for any particular purpose and does not accept any liability howsoever arising, including but not limited to negligence for any loss resulting from the use of or reliance upon the data. Data has been generated under a collaborative effort with the Baltimore Ecosystem Study (BES), affiliated organisations and researchers for the production of peer-reviewed scientific articles, book chapters, presentations, and other intellectual products. Any other commercial use is not allowed under any circumstance without expressed and written consent from the data creator. The data producer retains the intellectual property of the data and the algorithm used for its production, as well as derived data, products and publications.

REFERENCES

Ossola, A., Locke, D., Lin, B., Minor, E., 2019a. Greening in style: Urban form, architecture and the          structure of front and backyard vegetation. Landscape and Urban Planning, 185: 141-157. doi:          https://doi.org/10.1016/j.landurbplan.2019.02.014

Ossola, A., Locke, D., Lin, B., Minor, E., 2019b. Yards increase forest connectivity in urban landscapes.           Landscape Ecology. 34, 2935–2948. https://doi.org/10.1007/s10980-019-00923-7

Locke, D., Ossola, A., Minor, E., Lin, B., 2022. Spatial contagion structures urban vegetation from parcel to landscape. People and Nature, 4(1):88-102. https://doi.org/10.1002/pan3.10254

Locke, D., Ossola, A., Schmit, J.P., Grove, M., 2024. Sub-parcel scale analysis is needed to capture              socially-driven canopy cover change in Baltimore, MD. (under review)